10-14-25										Proposal

Montana Contracting,

173 N Route 9W

Congers, NY 10920

Re: South Orangetown Ambulance Corp, Tappan.

Scope of work – Concrete

- Form and pour new concrete footings (24” wide x 12” deep) at perimeter of new extension and interior walls as per Details on Drawing A-1 and A-8.

- Form & pour 2# 5’-0” x 4’-0” x 12” deep concrete footings as per Drawing A-1.

- Form & pour 2# 5’-0” x 4’-0” x 18” deep concrete footings as per Drawing A-1.

- Form & pour 1# 7’-0” x 6’-6” x 18” deep concrete footings as per Drawing A-1.

- Furnish & install new 6” deep 4000PSI concrete slab, reinforced with 6 x 6 10/10 WWF on 6 mil poly Vapor Barrier on 6” compacted crushed stone as per Drawings A-1 and A-8.

- Furnish & install 2” XPS insulation (R-10) over ¾” R-5 Polyiso continuous at the inside face of the exterior CMU foundation wall.

- Deliveries and clean up included.

TOTAL SUM FOR SCOPE OF WORK……………$115,685.00

Scope of work – Masonry.

- Furnish & install 8” exterior CMU on top of concrete perimeter footing as per Details on Drawing A-8 & A-9.

- Exterior CMU to be grouted solid.

- Reinforce exterior CMU with Truss type reinforcing every other course as per Drawing A-8.

- Furnish & install 2 course of CMU at interior walls as per note on Drawing A-1.

- Deliveries and clean up included.

TOTAL SUM FOR SCOPE OF WORK……………$53,295.00

Scope of work – Parging

- Furnish & install 3/8” cement parge on the exterior cmu from grade to top of wall as per Drawing A-8.

- Furnish & install Bituminous waterproofing on the exterior CMU wall from grade down to the concrete footing.

- Deliveries and clean up included.

TOTAL SUM FOR SCOPE OF WORK……………$29,335.00

Notes & Exclusions.

- All work on regular time.

- Price based on Non-union labor.

- Temp power & water supplied by GC.

- All excavation of footings etc. by others.

- Backfilling around new concrete/cmu footings/foundation walls by others.

- Latex paint at exterior parged walls by others.

- Welding excluded.

- Winter heat excluded.

Regards

__________________

Tony Donnelly